LaaS POC — Updated Runbook

GPU-as-a-Service Infrastructure — As Executed & Validated

March 2026  |  Version 2.0  |  Status: Phase 2 Complete

Purpose of this Document: This is the definitive as-executed runbook for the LaaS POC. It records exactly what was built, what deviated from the original plan, what was discovered, and what was validated. It serves as the primary handoff document for the software MVP builder who will build the orchestration platform, APIs, and web portal on top of this infrastructure. Every section marked 🔵 MVP BUILDER NOTE contains critical context for building the software layer.

# Hardware

## POC Test Machine (Single Node)

All POC validation was performed on a single machine. This is NOT one of the 4 production nodes. The GPU architecture differs — POC uses Ada Lovelace (RTX 4090), production will use Blackwell (RTX 5090). Behavioral validation must be re-run on production hardware.

| Component | POC Machine (Test) | Production Nodes (x4) |
| --- | --- | --- |
| CPU | AMD Ryzen 9 7950X3D (32 threads) | AMD Ryzen 9 9950X3D (32 threads, 3D V-Cache) |
| GPU | NVIDIA RTX 4090 24GB (Ada, sm_89) | Zotac RTX 5090 32GB (Blackwell, sm_100) |
| RAM | 64GB DDR5 | 64GB DDR5 G.SKILL 6000MHz |
| Storage | 2TB NVMe | Samsung 990 EVO Plus 2TB NVMe |
| OS | Ubuntu 22.04 LTS (bare metal) | Ubuntu 22.04 LTS (bare metal on Proxmox host) |
| Driver | 570.211.01 | To be installed — same version target |
| CUDA | 12.8.93 | 12.8 target |
| Remote | Tailscale VPN (100.100.66.101) | Cloudflare Tunnel (production) |

⚠ BLACKWELL NOTE: RTX 5090 is Blackwell (sm_100). All POC validation was on Ada (sm_89). NVENC concurrent stream limits, MPS fault propagation behavior, and the D3cold reset bug may differ. Explicit re-validation required on production nodes before launch.

# Architecture Overview

All user sessions — stateful GUI desktops and ephemeral CLI/Jupyter — run as Selkies EGL Desktop Docker containers. This is the unified container architecture (Architecture A) as defined in the LaaS Infrastructure Plan.

| Layer | Technology | Status |
| --- | --- | --- |
| Container Runtime | Docker CE 29.3.0 | ✅ COMPLETED |
| Desktop Streaming | Selkies-GStreamer + WebRTC (ghcr.io image) | ✅ COMPLETED |
| Desktop Environment | KDE Plasma 5.27 on Ubuntu 24.04 (inside container) | ✅ COMPLETED |
| GPU Sharing — VRAM | HAMi-core (libvgpu.so) — LD_PRELOAD interception | ✅ COMPLETED |
| GPU Sharing — SM | CUDA MPS (nvidia-cuda-mps-control daemon) | ✅ COMPLETED |
| Video Encoding | NVENC hardware encoding (nvh264enc via GStreamer) | ✅ COMPLETED |
| Persistent Storage | ZFS on NAS + NFS mount per user (/home/ubuntu) | ✅ COMPLETED |
| Resource Visibility | lxcfs for /proc spoofing (RAM confirmed, CPU partial) | ⚠ DEVIATION |
| Process Supervision | supervisord (custom config — critical, see Section 5) | ✅ COMPLETED |
| Remote Access | Tailscale (POC) → Cloudflare Tunnel (production) | ⬜ PENDING |
| Monitoring | Prometheus + Grafana + DCGM Exporter | ⬜ PENDING |
| Auth | Keycloak | ⬜ PENDING |

# Disk Layout

## POC Machine (Dual-Boot — Different from Production)

The POC machine was already dual-booting Windows 11 and Ubuntu 22.04 when the POC began. The production nodes will be clean installs. The POC layout deviates from the plan.

| Partition | Size | Purpose | Note |
| --- | --- | --- | --- |
| nvme0n1p1 | 100MB | /boot/efi | EFI |
| nvme0n1p2 | 16MB | MSR (Windows) | DO NOT TOUCH |
| nvme0n1p3 | 1.7TB | Windows C: | DO NOT TOUCH |
| nvme0n1p4 | 100GB | Ubuntu root (/) | POC only — too small for production |
| nvme0n1p5 | 822MB | Recovery |  |
| /vg_containers | — | Directory on root (NOT dedicated partition) | POC workaround |

## Production Node Target Layout (per node, 2TB)

nvme0n1p1   128GB   EXT4   →  OS + swap

nvme0n1p2   500GB   EXT4   →  /var/lib/docker  (images, overlays)

nvme0n1p3  1100GB   LVM PV →  VG_CONTAINERS (ephemeral scratch)

nvme0n1p4   240GB   EXT4   →  Shared read-only datasets

🔵 MVP BUILDER NOTE: On production nodes, Docker storage is on a dedicated 500GB partition, not the OS partition. Container writable overlays are ephemeral — destroyed on container stop. User persistent data lives entirely on NAS via NFS. The orchestrator never needs to manage local disk for user data.

# Pre-Flight Setup

## Step 0A — BIOS Configuration

Required settings verified before POC:

- AMD-Vi (IOMMU): Enabled

- SVM Mode (AMD-V): Enabled

- Resizable BAR / SAM: Enabled

- XMP/EXPO: Enabled (RAM stability)

- BIOS version 1710 → updated via ASUS EZ Flash 3 before POC

## Step 0B — NVIDIA Driver + CUDA

# Verify (already installed on POC machine):

nvidia-smi

# Expected: RTX 4090, Driver 570.211.01

nvcc --version

# Expected: CUDA 12.8.93

# If installing fresh:

sudo add-apt-repository ppa:graphics-drivers/ppa -y && sudo apt update

sudo apt install -y nvidia-driver-570 nvidia-utils-570

sudo reboot

# CUDA Toolkit 12.8:

wget https://developer.download.nvidia.com/compute/cuda/repos/ubuntu2204/x86_64/cuda-keyring_1.1-1_all.deb

sudo dpkg -i cuda-keyring_1.1-1_all.deb

sudo apt update && sudo apt install -y cuda-toolkit-12-8

## Step 0C — Docker CE + NVIDIA Container Toolkit

curl -fsSL https://get.docker.com | sudo bash

sudo usermod -aG docker $USER && newgrp docker

# NVIDIA Container Toolkit:

curl -fsSL https://nvidia.github.io/libnvidia-container/gpgkey | \

sudo gpg --dearmor -o /usr/share/keyrings/nvidia-container-toolkit-keyring.gpg

curl -s -L https://nvidia.github.io/libnvidia-container/stable/deb/nvidia-container-toolkit.list | \

sed 's#deb https://#deb [signed-by=...keyring.gpg] https://#g' | \

sudo tee /etc/apt/sources.list.d/nvidia-container-toolkit.list

sudo apt update && sudo apt install -y nvidia-container-toolkit

sudo nvidia-ctk runtime configure --runtime=docker

sudo systemctl restart docker

⚠ DEVIATION: The original plan used '--gpus all' for GPU passthrough. In practice, --device flags are more reliable with custom images. Both methods work. Use --gpus all with standard images, --device flags with cuda-hami-env.

⚠ DEVIATION: nvidia/cuda:12.8.0-base and runtime images do NOT include nvidia-smi binary. To test GPU in a clean container, mount nvidia-smi and libnvidia-ml.so.1 from the host into ubuntu:22.04 instead.

# HAMi-Core Build and Validation

## What HAMi-Core Does

HAMi-core provides VRAM and SM (compute) limits per container via LD_PRELOAD interception of CUDA API calls. It is NOT a kernel module or daemon — it is a shared library (libvgpu.so) that intercepts cuMemAlloc, cuMemGetInfo, and related calls.

Key mechanism: Each container that loads libvgpu.so via LD_PRELOAD creates or attaches to a file-backed shared memory region at /tmp/cudevshr.cache. This file stores the memory limits and per-process usage tracking. Limits are read from environment variables at init time.

## Step 1 — Build libvgpu.so

sudo apt install -y build-essential cmake git

git clone https://github.com/Project-HAMi/HAMi-core.git

cd HAMi-core && mkdir build && cd build

cmake .. -DCMAKE_BUILD_TYPE=Release

make -j$(nproc)

ls -la libvgpu.so   # Confirm: ~418784 bytes

# CORRECT install path (DEVIATION from original plan):

sudo cp libvgpu.so /usr/lib/libvgpu.so   # NOT /usr/local/lib/

sudo ldconfig

ls -la /usr/lib/libvgpu.so

⚠ CRITICAL DEVIATION: libvgpu.so must be at /usr/lib/libvgpu.so — NOT /usr/local/lib/. This is the correct path for bare metal Ubuntu. Using the wrong path causes silent failures where HAMi initializes but LD_PRELOAD is never resolved.

## Step 2 — CUDA MPS Daemon

# Stop any manually-started MPS first:

echo 'quit' | sudo nvidia-cuda-mps-control 2>/dev/null || true

sudo tee /etc/systemd/system/cuda-mps.service > /dev/null << 'EOF'

[Unit]

Description=CUDA MPS Control Daemon

After=nvidia-persistenced.service

[Service]

Type=forking

Environment=CUDA_VISIBLE_DEVICES=0

Environment=CUDA_MPS_PIPE_DIRECTORY=/tmp/nvidia-mps

Environment=CUDA_MPS_LOG_DIRECTORY=/tmp/nvidia-log

ExecStartPre=/usr/bin/nvidia-smi -pm 1

ExecStart=/usr/bin/nvidia-cuda-mps-control -d

ExecStop=/bin/bash -c "echo quit | /usr/bin/nvidia-cuda-mps-control"

Restart=on-failure

RestartSec=10

[Install]

WantedBy=multi-user.target

EOF

sudo systemctl daemon-reload

sudo systemctl enable --now cuda-mps

sudo systemctl status cuda-mps   # Expect: active (running)

⚠ NOTE: MPS get_server_list returning empty is NORMAL. Servers spawn on-demand when a CUDA process connects. This is not a failure.

## Step 3 — HAMi Enforcement Validation

This is the definitive test. Run inside a container with LD_PRELOAD set:

# Install torch inside container:

pip install torch --index-url https://download.pytorch.org/whl/cu128 -q --break-system-packages

# Run enforcement test:

python3 -c "

import torch

total = torch.cuda.get_device_properties(0).total_memory

print(f'Reported VRAM: {total/1024**3:.1f}GB')   # Should show 4.0GB

print('Trying to allocate 6GB (should FAIL if limit is 4GB)...')

try:

t = torch.zeros(int(6*1024**3/4), dtype=torch.float32, device='cuda')

print('FAIL: limit NOT enforced')

except RuntimeError as e:

print(f'SUCCESS: blocked — {e}')

print('Trying to allocate 3GB (should SUCCEED)...')

try:

t = torch.zeros(int(3*1024**3/4), dtype=torch.float32, device='cuda')

print('SUCCESS: 3GB allocated fine')

except RuntimeError as e:

print(f'FAIL: blocked at 3GB — unexpected')

"

| Test | Expected | Actual Result | Status |
| --- | --- | --- | --- |
| Reported VRAM (torch) | 4.0 GB | 4.0 GB | ✅ PASSED |
| 6GB allocation attempt | BLOCKED (OOM) | BLOCKED — HAMi OOM fired at 4294967296 bytes | ✅ PASSED |
| 3GB allocation attempt | SUCCEEDS | Allocated fine | ✅ PASSED |
| nvidia-smi VRAM display | Shows full 24GB (expected — NVML bypass) | Shows 24GB | ✅ PASSED |
| SM limit (25%) | Enforced via MPS ACTIVE_THREAD_PERCENTAGE | Set in cudevshr.cache | ✅ PASSED |

IMPORTANT — nvidia-smi always shows full GPU: nvidia-smi queries NVML directly at the kernel driver level. HAMi cannot intercept NVML calls — only CUDA API calls. This means nvidia-smi will always report the full GPU VRAM regardless of limits. This is expected behavior, not a bug. The actual enforcement (HAMi OOM) is real and works correctly. The recommended approach: wrap the nvidia-smi binary (see Section 7) and document this limitation in Terms of Service.

Technical detail — how HAMi stores limits: When a CUDA process with LD_PRELOAD loads, HAMi creates or attaches to /tmp/cudevshr.cache — a file-backed mmap'd shared region (struct shared_region_t, ~2MB). The limit[] array at offset ~1600 bytes holds per-device VRAM limits in bytes. The first process to create the file writes the limits from env vars. Subsequent processes in the same container attach to the same file and verify consistency. Each container gets its own isolated cache file (different Docker overlay filesystem). This is per-container and correct.

# Selkies EGL Desktop Image

## Image Details

| Property | Value |
| --- | --- |
| Image | ghcr.io/selkies-project/nvidia-egl-desktop:latest |
| Base OS inside container | Ubuntu 24.04 (NOT 22.04 — important for software installs) |
| Desktop | KDE Plasma 5.27.12 |
| Default user | ubuntu (uid 1000) |
| Default password | Set via PASSWD env var |
| Streaming | selkies-gstreamer (WebRTC, GStreamer pipeline) |
| Video encoder | nvh264enc (NVIDIA NVENC hardware encoding) |
| Port (internal) | Set via SELKIES_PORT env var (NOT fixed 8080) |
| Web proxy | nginx (listens on NGINX_PORT env var) |

⚠ DEVIATION: The plan called for building the Selkies image from source (20-40 min). In practice, the pre-built ghcr.io image was used — faster and more reliable. Build from source only when custom software needs to be baked in. The image uses Ubuntu 24.04 internally, not 22.04 as originally planned.

## Pulling the Image

docker pull ghcr.io/selkies-project/nvidia-egl-desktop:latest

docker images | grep selkies

# Pre-pull on ALL nodes before launch (production):

# This eliminates pull time from session startup latency

🔵 MVP BUILDER NOTE: The image must be pre-pulled on all 4 production nodes. Build a startup check in the orchestrator that verifies image availability on each node at boot. Do not launch containers on a node where the image is not yet pulled.

# Critical: supervisord Configuration

## Why This Configuration is Non-Negotiable

The Selkies container has multiple process trees that must NOT all have LD_PRELOAD set. VGL (VirtualGL) and the X server/KDE startup will crash or produce rendering errors if libvgpu.so is loaded globally. The supervisord config below is the result of extensive debugging and is the validated working solution.

| Process | LD_PRELOAD | Why |
| --- | --- | --- |
| entrypoint (X server, VGL, KDE) | NONE | VGL dlopen fails if libvgpu.so is in LD_PRELOAD |
| selkies-gstreamer (NVENC encoder) | INLINE in command only | Needs CUDA interception for encoder VRAM accounting |
| User Konsole/terminal sessions | Via /etc/bash.bashrc | Users need HAMi limits when running CUDA workloads |
| nvidia-smi binary | WRAPPED (see Section 7) | Prevents segfault from NVML + HAMi interaction |

## supervisord-hami.conf — Complete Working Config

Save this to /tmp/supervisord-hami.conf on the host and mount into every container:

cat > /tmp/supervisord-hami.conf << 'SUPERVISORD_EOF'

[unix_http_server]

file=/tmp/supervisor.sock

chmod=0700

[supervisord]

logfile=/tmp/supervisord.log

logfile_maxbytes=5MB

logfile_backups=0

loglevel=info

pidfile=/tmp/supervisord.pid

childlogdir=/tmp

nodaemon=true

[rpcinterface:supervisor]

supervisor.rpcinterface_factory = supervisor.rpcinterface:make_main_rpcinterface

[supervisorctl]

serverurl=unix:///tmp/supervisor.sock

[include]

files = /etc/supervisor/conf.d/*.conf

[program:entrypoint]

command=bash -c "dbus-run-session -- /etc/entrypoint.sh"

stdout_logfile=/tmp/entrypoint.log

stdout_logfile_maxbytes=5MB

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=1

[program:dbus]

command=bash -c "mkdir -pm700 \"${XDG_RUNTIME_DIR}\"; ...; dbus-daemon --system ..."

environment=DISPLAY=%(ENV_DISPLAY)s,...

# (see full config in repo)

[program:selkies-gstreamer]

# CRITICAL: LD_PRELOAD is set INLINE here, NOT in docker -e LD_PRELOAD

command=bash -c "if [ \"$(echo ${KASMVNC_ENABLE}...)\" != \"true\" ]; then \

LD_PRELOAD=/usr/lib/libvgpu.so /etc/selkies-gstreamer-entrypoint.sh; \

else sleep infinity; fi"

stdout_logfile=/tmp/selkies-gstreamer-entrypoint.log

stdout_logfile_maxbytes=5MB

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=20

# ... nginx, pipewire, kasmvnc programs follow

SUPERVISORD_EOF

## bash.bashrc Injection for User Shell Sessions

User terminal sessions (Konsole inside the desktop) are non-login shells. /etc/profile.d/ scripts are NOT sourced. LD_PRELOAD must be injected via /etc/bash.bashrc:

# Get original bashrc from image

docker run --rm --entrypoint cat \

ghcr.io/selkies-project/nvidia-egl-desktop:latest \

/etc/bash.bashrc > /tmp/bash.bashrc.orig

# Append HAMi injection

cat >> /tmp/bash.bashrc.orig << 'EOF'

# HAMi CUDA interception — applied to all bash sessions

# NOT set globally — only activates when user opens a terminal

# VGL/X/KDE startup is unaffected (uses /etc/entrypoint.sh, not login shells)

if [ -f /usr/lib/libvgpu.so ] && [ "${HAMI_INJECTED}" != "1" ]; then

export LD_PRELOAD=/usr/lib/libvgpu.so

export HAMI_INJECTED=1

mkdir -p /tmp/vgpulock 2>/dev/null

fi

EOF

chmod 644 /tmp/bash.bashrc.orig

⚠ DEVIATION: profile.d scripts are NOT sourced for non-login shells. Konsole opens non-login interactive shells. /etc/bash.bashrc is the correct injection point. This was discovered after multiple failed attempts with profile.d.

# Binary Wrappers

## nvidia-smi Wrapper

nvidia-smi segfaults when both libvgpu.so LD_PRELOAD and NVML are active simultaneously. Wrap it to strip LD_PRELOAD before execution:

cat > /tmp/nvidia-smi-wrapper << 'EOF'

#!/bin/bash

# Strip HAMi LD_PRELOAD before calling real nvidia-smi

exec env -u LD_PRELOAD /usr/bin/nvidia-smi.real "$@"

EOF

chmod +x /tmp/nvidia-smi-wrapper

🔵 MVP BUILDER NOTE: nvidia-smi will always show the full host GPU VRAM (e.g., 24GB for RTX 4090, 32GB for RTX 5090) regardless of container limits. This is a kernel driver limitation — NVML bypasses all user-space interception. The wrapper prevents segfaults. Do NOT attempt to spoof nvidia-smi output — focus on real enforcement (HAMi) and document the limitation transparently.

## passwd Wrapper

cat > /tmp/passwd-wrapper << 'EOF'

#!/bin/bash

# Strip LD_PRELOAD before calling real passwd (prevents segfault)

exec env -u LD_PRELOAD /usr/bin/passwd.real "$@"

EOF

chmod +x /tmp/passwd-wrapper

# NFS Persistent Storage Setup

## Architecture

Each user gets exactly 15GB of persistent storage as a ZFS dataset on the NAS, mounted via NFS to whichever node their session runs on. This is the source of truth for user data across all sessions.

| Property | Value |
| --- | --- |
| NAS filesystem | ZFS (TrueNAS Scale or zfsutils-linux) |
| Pool | datapool |
| User dataset path | datapool/users/<uid> |
| Quota | 15GB hard limit enforced at ZFS level |
| Mount point in container | /home/ubuntu |
| NFS protocol | NFSv4 over 10GbE |
| POC simulation | File-backed ZFS pool (20GB), 5GB quota per user (space constraint) |

## POC Setup Commands

sudo apt install -y zfsutils-linux nfs-kernel-server

# POC: file-backed pool (no dedicated partition available)

sudo mkdir -p /vg_containers

sudo truncate -s 20G /vg_containers/nas_pool.img

sudo zpool create -f datapool /vg_containers/nas_pool.img

# Create user volumes

sudo zfs create datapool/users

sudo zfs create -o quota=5G datapool/users/testuser1

sudo zfs create -o quota=5G datapool/users/testuser2

# NFS export (loopback for POC)

echo '/datapool/users 127.0.0.1(rw,sync,no_subtree_check,no_root_squash)' | sudo tee -a /etc/exports

sudo exportfs -ra

sudo systemctl restart nfs-kernel-server

# Mount

sudo mkdir -p /mnt/nfs/users

sudo mount -t nfs4 127.0.0.1:/datapool/users /mnt/nfs/users

# Create home dirs with correct ownership (uid 1000 = ubuntu inside container)

for i in 1 2 3 4; do

sudo mkdir -p /mnt/nfs/users/testuser$i

sudo chown 1000:1000 /mnt/nfs/users/testuser$i

done

⚠ DEVIATION: POC uses 20GB file-backed pool with 5GB quotas (not 15GB) due to root partition space constraints. Production uses dedicated NAS hardware with 15GB quotas per user as specified in the infra plan.

🔵 MVP BUILDER NOTE: The NFS path exposed inside containers is 127.0.0.1:/datapool/users/testuser1 — this leaks internal infrastructure details. In production, use a resolvable internal hostname (e.g., nas.internal) instead of 127.0.0.1, and use abstract user IDs as path components instead of usernames (e.g., /homes/u_a7f3c2). The orchestrator must provision a new ZFS dataset and NFS export when a new user registers, and mount it to /home/ubuntu in every session that user launches.

# Resource Visibility: lxcfs

## Problem

Without intervention, containers show the host's full hardware specs in /proc and KDE System Settings. A user on a 4-core/8GB tier sees the full 32-core/64GB host machine. This exposes internal infrastructure and is a trust/security issue.

## Solution: lxcfs

lxcfs is a FUSE filesystem that intercepts /proc reads and returns container-aware values matching Docker cgroup limits.

# Install and start on HOST (not inside containers):

sudo apt install -y lxcfs

sudo systemctl enable --now lxcfs

# Verify it provides all needed files:

ls /var/lib/lxcfs/proc/

# Expected: cpuinfo  diskstats  loadavg  meminfo  slabinfo  stat  swaps  uptime

| Resource | Without lxcfs | With lxcfs | Status |
| --- | --- | --- | --- |
| RAM (free -h) | Shows full host 62GB | Shows container limit (8GB) | ✅ COMPLETED |
| CPU count (nproc) | Shows host 32 cores | Needs --cpuset-cpus flag too | ⚠ DEVIATION |
| KDE System Info RAM | Shows host 62GB | Shows container 8GB | ✅ COMPLETED |
| KDE System Info CPU | Shows host 32 cores | Partial fix (see below) | ⚠ DEVIATION |
| df -h | Shows real host partitions | Not fixable via lxcfs (kernel mounts) | 🔵 FOR MVP |
| nvidia-smi VRAM | Shows full GPU always | Not fixable (NVML kernel bypass) | 🔵 FOR MVP |

CPU NOTE: lxcfs alone does not fix nproc. --cpus=4 limits CPU time (throttling) but all 32 cores remain visible. Add --cpuset-cpus=0-3 to docker run to restrict visibility to 4 physical cores. lxcfs then correctly reports 4 in /proc/cpuinfo.

🔵 MVP BUILDER NOTE: df -h will always show the container overlay device and NFS mount path. This is a kernel-level reality — containers see their actual mounts. Mitigation for production: (1) Use abstract NFS paths with no identifying information. (2) The overlay root line (Device: overlay) is normal and reveals nothing sensitive. (3) Do not expose an SSH shell to users — the desktop-only UX limits what df output can expose.

# Validated Container Launch Parameters

## Complete Working docker run Command

This is the exact validated command for launching one Selkies desktop session. All deviations and required flags are documented inline:

# Prepare per-container vgpulock dir (MUST be per-container, not shared):

sudo mkdir -p /tmp/vgpulock-$i && sudo chmod 777 /tmp/vgpulock-$i

docker run -d \

--name selkies-$SESSION_ID \

# Resource limits (adjust per tier):

--gpus all                          # GPU access

--cpus=4                            # CPU time limit

--cpuset-cpus=0-3                   # CPU visibility (lxcfs needs this for nproc)

--memory=8g                         # RAM hard limit

# IPC and networking:

--ipc=host                          # Required for Selkies shared memory (WebRTC)

--network=host                      # POC ONLY — required for WebRTC over Tailscale

--tmpfs /dev/shm:rw                 # Shared memory for WebRTC

# Display and streaming:

-e DISPLAY=:$DISPLAY_NUM            # Unique per container (e.g., :20, :21, :22, :23)

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc        # Hardware NVENC encoding

# Auth:

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=$TOKEN \  # Per-session random token

-e PASSWD=mypasswd                  # Desktop login password

# Port assignment (unique per container — --network=host requires this):

-e NGINX_PORT=$NGINX_P              # Web UI port (e.g., 8081, 8082, 8083, 8084)

-e SELKIES_PORT=$SELKIES_P          # Internal gstreamer port (e.g., 9081, 9082)

-e SELKIES_METRICS_HTTP_PORT=$METRICS_P \

# GPU:

-e CUDA_VISIBLE_DEVICES=0           # GPU index

-e CUDA_NVRTC_ARCH=89               # sm_89 for Ada (sm_100 for Blackwell RTX 5090)

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

# HAMi VRAM limits (adjust per tier):

-e CUDA_DEVICE_MEMORY_LIMIT_0=4096m    # 4GB VRAM limit for this container

-e CUDA_DEVICE_SM_LIMIT=25             # 25% SM (compute) limit

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT='0=4G' \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=25 \

# TURN server for WebRTC (required for remote access through NAT/firewall):

-e SELKIES_TURN_HOST=$TURN_HOST \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=$TURN_USER \

-e SELKIES_TURN_PASSWORD=$TURN_PASS \

-e SELKIES_TURN_PROTOCOL=tcp \

# Volume mounts:

-v /mnt/nfs/users/$USER_ID:/home/ubuntu \  # User persistent storage

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \  # HAMi library

-v /tmp/vgpulock-$i:/tmp/vgpulock \          # Per-container HAMi lock dir

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \  # Real nvidia-smi

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \   # Wrapper replaces it

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \  # Critical custom config

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \           # HAMi injection for terminals

# lxcfs /proc spoofing:

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

ghcr.io/selkies-project/nvidia-egl-desktop:latest

⚠ PRODUCTION NETWORK CHANGE: --network=host is used in the POC because Tailscale VPN is the remote access method and WebRTC requires direct port access. In production, Cloudflare Tunnel handles remote access — it terminates at the host and reverse-proxies to containers. This means --network=bridge can and SHOULD be used in production for proper network namespace isolation between containers. With --network=bridge, use -p $NGINX_P:8080 and configure the Cloudflare Tunnel to proxy to the container's exposed port.

# Tier Resource Configurations

🔵 MVP BUILDER NOTE: The orchestrator must map each user's subscription tier to a specific set of resource parameters at container launch time. The table below defines the validated parameter sets for each tier. These are passed as environment variables and Docker flags to docker run.

| Tier | CPUs | RAM | VRAM | SM% | GPU Access | Session Type |
| --- | --- | --- | --- | --- | --- | --- |
| Starter | 2 | 4GB | None | None | No | Stateful GUI |
| Standard | 4 | 8GB | None | None | No | Stateful GUI |
| Pro | 4 | 8GB | 4GB | 25% | Yes (shared) | Stateful GUI |
| Power | 6 | 16GB | 8GB | 35% | Yes (shared) | Stateful GUI |
| Max | 8 | 24GB | 16GB | 50% | Yes (shared) | Stateful GUI |
| Full Machine | 16 | 64GB | 32GB | 100% | Yes (exclusive) | Stateful GUI |
| Ephemeral CPU | 2-4 | 4-8GB | None | None | No | CLI/Jupyter |
| Ephemeral GPU | 4 | 8GB | 4-8GB | 25-35% | Yes (shared) | CLI/Jupyter |

HAMi env vars per tier: CUDA_DEVICE_MEMORY_LIMIT_0 (e.g., 4096m, 8192m, 16384m, 32768m), CUDA_DEVICE_SM_LIMIT (e.g., 25, 35, 50, 100), CUDA_MPS_PINNED_DEVICE_MEM_LIMIT (e.g., '0=4G'), CUDA_MPS_ACTIVE_THREAD_PERCENTAGE (same as SM_LIMIT).

# Multi-Container Port Assignment

## The Port Problem and Solution

With --network=host, all containers share the host network namespace. Multiple containers cannot all bind to port 8080. Each container must be assigned unique ports via environment variables.

| Container Slot | NGINX_PORT (Web UI) | SELKIES_PORT (Internal) | METRICS_PORT | DISPLAY |
| --- | --- | --- | --- | --- |
| 1 | 8081 | 9081 | 19081 | :20 |
| 2 | 8082 | 9082 | 19082 | :21 |
| 3 | 8083 | 9083 | 19083 | :22 |
| 4 | 8084 | 9084 | 19084 | :23 |
| N (general) | 8080+N | 9080+N | 19080+N | :19+N |

⚠ DEVIATION: The original plan used -p 8080:8080 port mapping. This causes 'Connection failed' in WebRTC when using --network=host. With --network=host, -p flags are ignored. Use SELKIES_PORT and NGINX_PORT env vars only. In production with --network=bridge, revert to -p $NGINX_P:$INTERNAL_PORT.

🔵 MVP BUILDER NOTE: The orchestrator port registry: maintain a mapping of (node_id, slot_number) → port assignments. When launching a container, assign the next available slot on the target node. On container stop, free the slot. With --network=bridge in production, port conflicts are eliminated — each container has its own network namespace.

# Session Lifecycle for Orchestrator

🔵 MVP BUILDER NOTE: This section defines exactly what the orchestrator must do at each stage of a user session. These are the infrastructure operations — the software layer calls these.

## Session Launch Sequence

- Check resource availability on target node (VRAM budget, CPU slots, RAM headroom)

- Provision NFS home dir if first session for this user: zfs create -o quota=15G datapool/users/$uid

- Create per-container vgpulock dir: mkdir -p /tmp/vgpulock-$session_id && chmod 777

- Generate session token: TOKEN=$(openssl rand -hex 16)

- Assign port slot (NGINX_PORT, SELKIES_PORT, METRICS_PORT, DISPLAY number)

- Execute docker run with all parameters from Section 9

- Poll container health: wait for nginx to listen on NGINX_PORT (check every 2s, timeout 60s)

- Return session URL and token to frontend when ready (~15-25 seconds)

## Session Health Check

# Check if nginx is up (container ready for connection):

ss -tlnp | grep :$NGINX_PORT

# Or via docker inspect:

docker inspect --format='{{.State.Status}}' selkies-$SESSION_ID

# Check selkies-gstreamer is running (streaming ready):

docker exec selkies-$SESSION_ID grep 'RUNNING' /tmp/selkies-gstreamer-entrypoint.log | tail -1

## Session Termination

- Send graceful warning to user (15-min, 5-min before scheduled end)

- docker stop selkies-$SESSION_ID  (SIGTERM → graceful KDE shutdown → 30s timeout → SIGKILL)

- docker rm selkies-$SESSION_ID  (destroys writable overlay — all system-level changes lost)

- User NFS home dir is preserved (persists indefinitely)

- Free port slot in orchestrator registry

- Free VRAM budget on node

- Record session end, duration, resource usage for billing

## Session Reconnection

Container continues running if browser closes mid-session (until scheduled end time). The orchestrator returns the same NGINX_PORT and token to the user when they reconnect — WebRTC re-establishes automatically.

## Container Startup Time

Current measured cold-start time: ~15-25 seconds to KDE Plasma fully visible in browser. Primary bottleneck is KDE Plasma initialization. Future optimization: pre-warm idle containers per node so users get instant sessions.

# Monitoring Stack (Bonus — Partially Implemented)

cat > prometheus.yml << 'EOF'

global:

scrape_interval: 15s

scrape_configs:

- job_name: 'dcgm'

static_configs:

- targets: ['localhost:9400']

EOF

cat > docker-compose-monitoring.yml << 'EOF'

version: '3.8'

services:

dcgm-exporter:

image: nvcr.io/nvidia/k8s/dcgm-exporter:latest

runtime: nvidia

environment:

NVIDIA_VISIBLE_DEVICES: all

ports: ['9400:9400']

restart: always

prometheus:

image: prom/prometheus:latest

volumes: ['./prometheus.yml:/etc/prometheus/prometheus.yml']

ports: ['9090:9090']

restart: always

grafana:

image: grafana/grafana:latest

ports: ['3000:3000']

environment:

GF_SECURITY_ADMIN_PASSWORD: admin

restart: always

EOF

docker compose -f docker-compose-monitoring.yml up -d

# Phase 2 — Final Validation Checklist (Step 2.13)

| Component | Pass Condition | Status | Notes |
| --- | --- | --- | --- |
| GPU Driver | RTX 4090, 570.x driver | ✅ PASSED | 570.211.01 |
| CUDA | nvcc shows 12.8.x | ✅ PASSED | 12.8.93 |
| Docker GPU | Container sees GPU | ✅ PASSED | 24564MiB |
| HAMi-core build | libvgpu.so at /usr/lib/ | ✅ PASSED | 418784 bytes |
| HAMi VRAM reporting | torch reports limited VRAM | ✅ PASSED | 4.0GB reported (4GB tier) |
| HAMi VRAM enforcement | OOM at limit | ✅ PASSED | 6GB blocked, 3GB allowed |
| HAMi escape resistance | Single-shot + chunk allocation blocked | ✅ PASSED |  |
| HAMi stateless isolation | No state bleed between containers | ✅ PASSED | Per-container cudevshr.cache |
| MPS Daemon | systemctl shows active/running | ✅ PASSED |  |
| Selkies Image | ghcr.io image pulled | ✅ PASSED | Ubuntu 24.04 base |
| Single Selkies Session | Browser connects, KDE Plasma visible | ✅ PASSED |  |
| GPU in Selkies | CUDA works inside desktop | ✅ PASSED |  |
| NVENC Encoding | nvh264enc active in logs | ✅ PASSED |  |
| supervisord HAMi config | VGL clean, HAMi in gstreamer, terminals | ✅ PASSED | No VGL dlopen errors |
| NFS Home Dir | Files persist after container stop | ✅ PASSED | ZFS dataset on loopback pool |
| lxcfs RAM spoofing | /proc/meminfo shows container limit | ✅ PASSED | 8GB shown correctly |
| lxcfs CPU spoofing | nproc shows container CPU count | ⚠ DEVIATION | Needs --cpuset-cpus flag added |
| 4 Concurrent Sessions | All 4 accessible with correct limits | ⬜ PENDING |  |
| Container Fault Isolation | Container 1 crash, others survive | ⬜ PENDING |  |
| Container Restart | NFS home intact after restart | ⬜ PENDING |  |

# Known Limitations and Production Considerations

## What Requires Production Validation

- RTX 5090 (Blackwell sm_100) — all tests were on RTX 4090 (Ada sm_89). NVENC concurrent limits, HAMi behavior, MPS fault propagation must be re-validated.

- CUDA_NVRTC_ARCH must change from 89 to 100 for Blackwell in all container launch commands.

- The D3cold GPU reset bug (Proxmox forum, NVIDIA forums) — RTX 5090 specific. Must test VM passthrough stability if Architecture B is ever used.

## Security Gaps to Address Before Production

- --network=host in POC: Switch to --network=bridge in production with Cloudflare Tunnel. This is the most critical security fix — --network=host means all containers share the host network namespace.

- --ipc=host: Required for Selkies WebRTC shared memory. Evaluate whether --ipc=private breaks WebRTC in production environment.

- df -h leaks host partition info: Use abstract NFS paths (hostname not IP, obfuscated user IDs).

- nvidia-smi always shows full GPU: Document in ToS. Cannot be fixed at kernel level.

- Kata Containers (optional upgrade path): Provides genuine VM-level isolation (own kernel, own /proc) while keeping Docker API. Requires KVM validation on Proxmox nodes. Adds ~2-3 seconds to startup. Investigate for v2.

# MVP Software Builder: Key Facts Summary

If you are the engineer building the orchestration platform, APIs, and web portal on top of this infrastructure, here is everything you need to know:

| What You Need to Know | Details |
| --- | --- |
| How to launch a session | docker run with the exact parameters in Section 9. No Kubernetes, no Swarm — direct Docker API. |
| How to check if session is ready | Poll for nginx listening on NGINX_PORT. Typically ready in 15-25 seconds. |
| How to stop a session | docker stop → docker rm. NFS home persists. Everything else is ephemeral. |
| How to provision user storage | zfs create -o quota=15G datapool/users/$uid on NAS. Mount via NFS at container launch. |
| How VRAM limits work | env vars at docker run time → HAMi reads them at first CUDA call → writes to /tmp/cudevshr.cache. Limits are real and enforced. |
| Why nvidia-smi shows full GPU | NVML kernel bypass. Cannot be fixed. Document in ToS. Real enforcement is HAMi. |
| Port assignment | NGINX_PORT=8080+slot, SELKIES_PORT=9080+slot, DISPLAY=:19+slot. Unique per container on same node. |
| Multi-node session routing | NFS home follows user to any node. Orchestrator picks node with available VRAM budget. Return correct host IP + port to frontend. |
| lxcfs requirement | Must be installed and running on every host node. Required for RAM/CPU visibility in containers. |
| Pre-built files needed on host | supervisord-hami.conf, bash.bashrc.orig, nvidia-smi-wrapper, passwd-wrapper, libvgpu.so. These must be deployed to /tmp/ on each node before any sessions launch. |
| Session startup latency | ~15-25 seconds cold start. Pre-warming pool recommended for production UX. |
| The Selkies image OS | Ubuntu 24.04 internally (not 22.04). Software installs inside container use apt for Ubuntu 24.04 packages. |

End of Document — LaaS POC Runbook v2.0

Learnings / Notes

Working docker command! (reference)

docker stop selkies-test-1 && docker rm selkies-test-1

TOKEN=$(openssl rand -hex 16)

SESSION_HOST="ws-$(openssl rand -hex 4)"

echo "Access: http://100.100.66.101:8081 | token: $TOKEN | hostname: $SESSION_HOST"

docker run -d \

--name selkies-test-1 \

--hostname $SESSION_HOST \

--gpus all \

--cpus=4 \

--cpuset-cpus=0-3 \

--memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC \

-e DISPLAY=:20 \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=$TOKEN \

-e NGINX_PORT=8081 \

-e SELKIES_PORT=9081 \

-e SELKIES_METRICS_HTTP_PORT=19081 \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 \

-e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0=4096m \

-e CUDA_DEVICE_SM_LIMIT=25 \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="0=4G" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=25 \

-e SELKIES_TURN_HOST=106.51.247.170 \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=selkies \

-e SELKIES_TURN_PASSWORD=wVIAbfwkgkxjaCiZVX4BDsdU \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /mnt/nfs/users/testuser1:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /tmp/vgpulock-1:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

ghcr.io/selkies-project/nvidia-egl-desktop:latest

Static SM Partitioning

Step 1 — Update MPS daemon with static partitioning:

bash

sudo systemctl stop cuda-mps

sudo tee /etc/systemd/system/cuda-mps.service > /dev/null << 'EOF'

[Unit]

Description=CUDA MPS Control Daemon — Static SM Partitioning

After=nvidia-persistenced.service

[Service]

Type=forking

Environment=CUDA_VISIBLE_DEVICES=0

Environment=CUDA_MPS_PIPE_DIRECTORY=/tmp/nvidia-mps

Environment=CUDA_MPS_LOG_DIRECTORY=/tmp/nvidia-log

ExecStartPre=/usr/bin/nvidia-smi -pm 1

ExecStart=/usr/bin/nvidia-cuda-mps-control -d

ExecStop=/bin/bash -c "echo quit | /usr/bin/nvidia-cuda-mps-control"

Restart=on-failure

RestartSec=10

[Install]

WantedBy=multi-user.target

EOF

sudo systemctl daemon-reload

sudo systemctl enable --now cuda-mps

sudo systemctl status cuda-mps

Step 2 — Helper script for orchestrator to calculate SM% from VRAM:

bash

sudo tee /usr/local/bin/calc-sm-percent << 'EOF'

#!/bin/bash

# Usage: calc-sm-percent <container_vram_gb> <total_gpu_vram_gb>

# Returns: integer SM% to use for CUDA_MPS_ACTIVE_THREAD_PERCENTAGE

#

# RTX 4090 POC:  total_gpu_vram_gb = 24

# RTX 5090 PROD: total_gpu_vram_gb = 32

CONTAINER_VRAM=${1:?Usage: calc-sm-percent <container_vram_gb> <total_gpu_vram_gb>}

TOTAL_VRAM=${2:?Usage: calc-sm-percent <container_vram_gb> <total_gpu_vram_gb>}

# Integer math: floor(vram/total * 100), minimum 5%

PCT=$(( (CONTAINER_VRAM * 100) / TOTAL_VRAM ))

[ $PCT -lt 5 ] && PCT=5

echo $PCT

EOF

chmod +x /usr/local/bin/calc-sm-percent

# Verify the math

echo "=== SM% by VRAM tier (RTX 4090, 24GB pool) ==="

for vram in 2 4 6 8 12 16 24; do

pct=$(calc-sm-percent $vram 24)

echo "  ${vram}GB VRAM → ${pct}% SMs"

done

echo ""

echo "=== SM% by VRAM tier (RTX 5090, 32GB pool) ==="

for vram in 4 8 16 32; do

pct=$(calc-sm-percent $vram 32)

echo "  ${vram}GB VRAM → ${pct}% SMs"

done

Step 3 — Stop all containers and relaunch with proportional SM%:

bash

docker stop selkies-test-1 selkies-test-2 selkies-test-3 selkies-test-4

docker rm selkies-test-1 selkies-test-2 selkies-test-3 selkies-test-4

# Total VRAM pool on RTX 4090

TOTAL_VRAM_GB=24

for i in 1 2 3 4; do

NGINX_P=$((8080 + i))

SELKIES_P=$((9080 + i))

METRICS_P=$((19080 + i))

DISPLAY_N=$((19 + i))

CPUSET="$(( ($i-1)*4 ))-$(( ($i-1)*4+3 ))"

# VRAM per tier

VRAM_GB=$([ $i -le 2 ] && echo "4" || echo "8")

VRAM="${VRAM_GB}192m"  # in MB for HAMi

VRAM_MPS="0=${VRAM_GB}G"

# SM% proportional to VRAM

SM_PCT=$(calc-sm-percent $VRAM_GB $TOTAL_VRAM_GB)

SESSION_HOST="ws-$(openssl rand -hex 4)"

TOKEN=$(openssl rand -hex 16)

echo "Container $i → VRAM: ${VRAM_GB}GB | SM: ${SM_PCT}% | cpuset: $CPUSET | port: $NGINX_P | token: $TOKEN"

docker run -d \

--name selkies-test-$i \

--hostname $SESSION_HOST \

--gpus all \

--cpus=4 \

--cpuset-cpus=$CPUSET \

--memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC \

-e DISPLAY=:$DISPLAY_N \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=$TOKEN \

-e NGINX_PORT=$NGINX_P \

-e SELKIES_PORT=$SELKIES_P \

-e SELKIES_METRICS_HTTP_PORT=$METRICS_P \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 \

-e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0="${VRAM_GB}192m" \

-e CUDA_DEVICE_SM_LIMIT=$SM_PCT \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="$VRAM_MPS" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=$SM_PCT \

-e CUDA_MPS_ENABLE_PER_CTX_DEVICE_MULTIPROCESSOR_PARTITIONING=1 \

-e SELKIES_TURN_HOST=106.51.247.170 \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=selkies \

-e SELKIES_TURN_PASSWORD=wVIAbfwkgkxjaCiZVX4BDsdU \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /mnt/nfs/users/testuser$i:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /tmp/vgpulock-$i:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

ghcr.io/selkies-project/nvidia-egl-desktop:latest

done

```

Paste the output of the container launch lines — should show:

```

Container 1 → VRAM: 4GB | SM: 16% | ...

Container 2 → VRAM: 4GB | SM: 16% | ...

Container 3 → VRAM: 8GB | SM: 33% | ...

Container 4 → VRAM: 8GB | SM: 33% | ...

TOKEN=$(openssl rand -hex 16)

echo "Access: http://100.100.66.101:8083 | token: $TOKEN"

Full start!!

docker stop selkies-test-1 selkies-test-2 selkies-test-3 selkies-test-4

docker rm selkies-test-1 selkies-test-2 selkies-test-3 selkies-test-4

TOTAL_VRAM_GB=24

for i in 1 2 3 4; do

NGINX_P=$((8080 + i))

SELKIES_P=$((9080 + i))

METRICS_P=$((19080 + i))

DISPLAY_N=$((19 + i))

CPUSET="$(( ($i-1)*4 ))-$(( ($i-1)*4+3 ))"

VRAM_GB=$([ $i -le 2 ] && echo "4" || echo "8")

SM_PCT=$(calc-sm-percent $VRAM_GB $TOTAL_VRAM_GB)

SESSION_HOST="ws-$(openssl rand -hex 4)"

TOKEN=$(openssl rand -hex 16)

MEM_BYTES=$((8*1024*1024*1024))

echo "Container $i → VRAM: ${VRAM_GB}GB | SM: ${SM_PCT}% | cpuset: $CPUSET | port: $NGINX_P | token: $TOKEN"

docker run -d \

--name selkies-test-$i \

--hostname $SESSION_HOST \

--gpus all \

--cpus=4 \

--cpuset-cpus=$CPUSET \

--memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC \

-e DISPLAY=:$DISPLAY_N \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=$TOKEN \

-e NGINX_PORT=$NGINX_P \

-e SELKIES_PORT=$SELKIES_P \

-e SELKIES_METRICS_HTTP_PORT=$METRICS_P \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 \

-e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0="${VRAM_GB}192m" \

-e CUDA_DEVICE_SM_LIMIT=$SM_PCT \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="0=${VRAM_GB}G" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=$SM_PCT \

-e CUDA_MPS_ENABLE_PER_CTX_DEVICE_MULTIPROCESSOR_PARTITIONING=1 \

-e CONTAINER_MEMORY_LIMIT_BYTES=$MEM_BYTES \

-e SELKIES_TURN_HOST=106.51.247.170 \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=selkies \

-e SELKIES_TURN_PASSWORD=wVIAbfwkgkxjaCiZVX4BDsdU \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /mnt/nfs/users/testuser$i:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /usr/lib/fake_sysconf.so:/usr/lib/fake_sysconf.so \

-v /tmp/vgpulock-$i:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

-v /tmp/container-$i-cpu:/sys/devices/system/cpu:ro \

ghcr.io/selkies-project/nvidia-egl-desktop:latest

done

docker run -d \

--name selkies-test-2 \

--hostname ws-$(openssl rand -hex 4) \

--gpus all \

--cpus=4 \

--cpuset-cpus=0-3 \

--memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC \

-e DISPLAY=:21 \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=123 \

-e NGINX_PORT=8082 \

-e SELKIES_PORT=9082 \

-e SELKIES_METRICS_HTTP_PORT=19082 \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 \

-e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0=4096m \

-e CUDA_DEVICE_SM_LIMIT=16 \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="0=4G" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=16 \

-e CUDA_MPS_ENABLE_PER_CTX_DEVICE_MULTIPROCESSOR_PARTITIONING=1 \

-e CONTAINER_MEMORY_LIMIT_BYTES=$((8*1024*1024*1024)) \

-e SELKIES_TURN_HOST=106.51.247.170 \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=selkies \

-e SELKIES_TURN_PASSWORD=wVIAbfwkgkxjaCiZVX4BDsdU \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /mnt/nfs/users/testuser2:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /usr/lib/fake_sysconf.so:/usr/lib/fake_sysconf.so \

-v /tmp/vgpulock-2:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

-v /tmp/container-2-cpu:/sys/devices/system/cpu:ro \

--label laas.session_id=$(openssl rand -hex 8) \

--label laas.user_id=testuser2@laas.in \

--label laas.tier=pro \

--label laas.session_type=stateful \

--label laas.node=node-mvp \

ghcr.io/selkies-project/nvidia-egl-desktop:latest



Correct Docker command: (prior nvidia-smi type -> C)
To use MPS

-v /tmp/nvidia-mps:/tmp/nvidia-mps \

-v /tmp/nvidia-log:/tmp/nvidia-log \

docker run -d \

--name selkies-test-2 \

--hostname ws-$(openssl rand -hex 4) \

--gpus all \

--cpus=4 \

--cpuset-cpus=0-3 \

--memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC \

-e DISPLAY=:21 \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=123 \

-e NGINX_PORT=8082 \

-e SELKIES_PORT=9082 \

-e SELKIES_METRICS_HTTP_PORT=19082 \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 \

-e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0=4096m \

-e CUDA_DEVICE_SM_LIMIT=16 \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="0=4G" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=16 \

-e CUDA_MPS_ENABLE_PER_CTX_DEVICE_MULTIPROCESSOR_PARTITIONING=1 \

-e CONTAINER_MEMORY_LIMIT_BYTES=$((8*1024*1024*1024)) \

-e SELKIES_TURN_HOST=106.51.247.170 \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=selkies \

-e SELKIES_TURN_PASSWORD=wVIAbfwkgkxjaCiZVX4BDsdU \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /tmp/nvidia-mps:/tmp/nvidia-mps \

-v /tmp/nvidia-log:/tmp/nvidia-log \

-v /mnt/nfs/users/testuser2:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /usr/lib/fake_sysconf.so:/usr/lib/fake_sysconf.so \

-v /tmp/vgpulock-2:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

-v /tmp/container-2-cpu:/sys/devices/system/cpu:ro \

--label laas.session_id=$(openssl rand -hex 8) \

--label laas.user_id=testuser2@laas.in \

--label laas.tier=pro \

--label laas.session_type=stateful \

--label laas.node=node-mvp \

ghcr.io/selkies-project/nvidia-egl-desktop:latest



cAdvisor is set on the host!
docker run -d \

--name cadvisor \

--privileged \

-p 8999:8080 \

-v /:/rootfs:ro \

-v /var/run:/var/run:rw \

-v /var/lib/docker:/var/lib/docker:ro \

gcr.io/cadvisor/cadvisor:v0.52.1 \

-housekeeping_interval=15s

Restart Policy

important!!!

CoreDump due to conflict between bash.rc and Hami!
You removed bash.bashrc mount → LD_PRELOAD never gets set → HAMi never loads → full 23.5GB visible.

The segfault was because your /tmp/bash.bashrc.orig was generated from the old cached image. You pulled a new image — the bashrc format changed slightly. Regenerate it from the new image:

# Regenerate bash.bashrc from the current image

docker run --rm --entrypoint cat \

ghcr.io/selkies-project/nvidia-egl-desktop:latest \

/etc/bash.bashrc > /tmp/bash.bashrc.orig

# Append HAMi + fake_sysconf injection

cat >> /tmp/bash.bashrc.orig << 'EOF'

# LaaS resource interceptors

if [ "${HAMI_INJECTED}" != "1" ]; then

export LD_PRELOAD="/usr/lib/libvgpu.so /usr/lib/fake_sysconf.so"

export HAMI_INJECTED=1

export SYSCONF_INJECTED=1

mkdir -p /tmp/vgpulock 2>/dev/null

fi

EOF

chmod 644 /tmp/bash.bashrc.orig

# Verify injection is in there

tail -10 /tmp/bash.bashrc.orig

stop and relaunch!!

What went wrong when I decided to delete user 1 and user 2 to expand 2 and 3’s disk alloc

sudo zpool create -f datapool /vg_containers/nas_pool_30g.img

zpool create with -f on a file that already has a pool wipes it and creates a fresh empty pool. The cp in step 4 copied the data correctly, but then zpool create destroyed it immediately after.

The correct command should have been zpool import, not zpool create.