LaaS Node

Complete Installation & Setup Guide

Bare Metal Ubuntu 22.04 → Fully Validated GPU Session Node

March 2026  |  Version 1.0  |  Based on POC Runbook v2.0

About this Guide: This is the complete, as-executed installation guide for a LaaS GPU session node. Every command has been run and validated on the POC machine. Commands are presented in exact execution order. All deviations from standard expectations are flagged. This guide takes you from a fresh Ubuntu 22.04 installation to a fully running, multi-session GPU desktop node.

# Hardware Reference

| Component | POC / Test Node | Production Node (×4) |
| --- | --- | --- |
| CPU | AMD Ryzen 9 7950X3D (32T) | AMD Ryzen 9 9950X3D (32T, 3D V-Cache) |
| GPU | NVIDIA RTX 4090 24GB (Ada, sm_89) | Zotac RTX 5090 32GB (Blackwell, sm_100) |
| RAM | 64GB DDR5 | 64GB DDR5 G.SKILL 6000MHz |
| Storage | 2TB NVMe (shared with Windows) | Samsung 990 EVO Plus 2TB NVMe (clean) |
| OS | Ubuntu 22.04 LTS (dual-boot) | Ubuntu 22.04 LTS (clean install) |
| CUDA arch flag | CUDA_NVRTC_ARCH=89 | CUDA_NVRTC_ARCH=100 |
| Total GPU VRAM pool | 24GB | 32GB |

⚠ WARNING: RTX 5090 is Blackwell (sm_100). All validation was on RTX 4090 (Ada, sm_89). Change CUDA_NVRTC_ARCH=89 to CUDA_NVRTC_ARCH=100 in all container launch commands on production nodes. Re-validate HAMi enforcement and NVENC encoding on first production node before fleet deployment.

STEP 1    BIOS Configuration

Before touching the OS, verify these BIOS settings. On AMD platform (ASUS/MSI/Gigabyte), enter BIOS at POST with DEL or F2.

| Setting | Value | Why |
| --- | --- | --- |
| AMD-Vi / IOMMU | Enabled | Required for GPU isolation and container GPU passthrough |
| SVM Mode (AMD-V) | Enabled | Hardware virtualisation — required for KVM/Kata if used later |
| Resizable BAR / SAM | Enabled | GPU performance — allows CPU full access to GPU memory |
| XMP / EXPO | Enabled | RAM stability at rated speed (6000MHz) |
| Above 4G Decoding | Enabled | Required for large GPU BAR mapping |
| BIOS version | Latest available | Update before OS install — use EZ Flash 3 / M-Flash |

ℹ INFO: On ASUS boards: AI Overclocking → Enabled for XMP/EXPO. AMD CBS → IOMMU = Enabled. Update BIOS using EZ Flash 3 from USB before starting — do this with Windows still on the machine if dual-booting.

STEP 2    Ubuntu 22.04 Post-Install Baseline

After clean Ubuntu 22.04 installation, run these baseline commands before anything else.

### 2.1 — System Update

sudo apt update && sudo apt upgrade -y

sudo apt install -y curl wget git build-essential cmake

sudo apt install -y pkg-config software-properties-common apt-transport-https

sudo apt install -y ca-certificates gnupg lsb-release htop nvtop net-tools

sudo reboot

### 2.2 — Verify Hardware Post-Reboot

# Verify CPU

lscpu | grep -E 'Model name|CPU\(s\)|Thread'

# Verify RAM

free -h

# Verify storage

lsblk

df -h /

# Verify IOMMU is enabled

dmesg | grep -i iommu | head -5

# Expected: AMD-Vi: AMD IOMMUv2 loaded and initialized

STEP 3    NVIDIA Driver + CUDA Installation

### 3.1 — Remove Any Existing NVIDIA Packages

sudo apt purge -y nvidia* libnvidia* cuda* 2>/dev/null || true

sudo apt autoremove -y

sudo reboot

### 3.2 — Install NVIDIA Driver 570

sudo add-apt-repository ppa:graphics-drivers/ppa -y

sudo apt update

sudo apt install -y nvidia-driver-570 nvidia-utils-570

sudo reboot

### 3.3 — Verify Driver

nvidia-smi

# Expected output:

# NVIDIA-SMI 570.x.x   Driver Version: 570.x.x   CUDA Version: 12.8

# GPU 0: NVIDIA GeForce RTX 4090 (or RTX 5090 on production)

# Verify persistence mode

sudo nvidia-smi -pm 1

nvidia-smi | grep Persistence

# Expected: Persistence-M: On

### 3.4 — Install CUDA Toolkit 12.8

wget https://developer.download.nvidia.com/compute/cuda/repos/ubuntu2204/x86_64/cuda-keyring_1.1-1_all.deb

sudo dpkg -i cuda-keyring_1.1-1_all.deb

sudo apt update

sudo apt install -y cuda-toolkit-12-8

# Add CUDA to PATH

echo 'export PATH=/usr/local/cuda/bin:$PATH' >> ~/.bashrc

echo 'export LD_LIBRARY_PATH=/usr/local/cuda/lib64:$LD_LIBRARY_PATH' >> ~/.bashrc

source ~/.bashrc

# Verify

nvcc --version

# Expected: release 12.8

⚠ WARNING: nvidia/cuda Docker images (e.g., nvidia/cuda:12.8.0-base) do NOT include nvidia-smi binary. Do not use these images for GPU testing. Use the Selkies image or mount nvidia-smi from host into ubuntu:22.04 for isolated tests.

STEP 4    Docker CE + NVIDIA Container Toolkit

### 4.1 — Install Docker CE

curl -fsSL https://get.docker.com | sudo bash

sudo usermod -aG docker $USER

newgrp docker

# Verify

docker --version

docker run --rm hello-world

### 4.2 — Install NVIDIA Container Toolkit

curl -fsSL https://nvidia.github.io/libnvidia-container/gpgkey | \

sudo gpg --dearmor -o /usr/share/keyrings/nvidia-container-toolkit-keyring.gpg

curl -s -L https://nvidia.github.io/libnvidia-container/stable/deb/nvidia-container-toolkit.list | \

sed 's#deb https://#deb [signed-by=/usr/share/keyrings/nvidia-container-toolkit-keyring.gpg] https://#g' | \

sudo tee /etc/apt/sources.list.d/nvidia-container-toolkit.list

sudo apt update

sudo apt install -y nvidia-container-toolkit

sudo nvidia-ctk runtime configure --runtime=docker

sudo systemctl restart docker

# Verify GPU is accessible in a container

docker run --rm --gpus all ubuntu:22.04 bash -c \

'apt-get install -y pciutils -q 2>/dev/null && lspci | grep -i nvidia'

✅ VALIDATED: Container sees GPU confirmed when you see the RTX 4090/5090 in lspci output.

STEP 5    HAMi-Core Build (VRAM + SM Enforcement)

HAMi-core compiles to libvgpu.so — a shared library that intercepts CUDA API calls to enforce per-container VRAM and SM limits. No kernel module, no daemon. Pure LD_PRELOAD interception.

### 5.1 — Build libvgpu.so

cd ~

git clone https://github.com/Project-HAMi/HAMi-core.git

cd HAMi-core

mkdir build && cd build

cmake .. -DCMAKE_BUILD_TYPE=Release

make -j$(nproc)

# Verify build

ls -la libvgpu.so

# Expected: ~418784 bytes

# Install to correct path

sudo cp libvgpu.so /usr/lib/libvgpu.so

sudo ldconfig

ls -la /usr/lib/libvgpu.so

🔴 CRITICAL: Install path MUST be /usr/lib/libvgpu.so — NOT /usr/local/lib/. Wrong path causes silent failures where HAMi loads but LD_PRELOAD is never resolved inside containers.

### 5.2 — Backup (before any future patching)

sudo cp /usr/lib/libvgpu.so /usr/lib/libvgpu.so.backup

ls -la /usr/lib/libvgpu.so.backup

ℹ INFO: If you later build from the patched fork (loiht2/HAMi-core-fix-memory) for improved nvidia-smi NVML interception, restore from this backup if anything breaks.

STEP 6    CUDA MPS Daemon (Compute Sharing)

MPS (Multi-Process Service) allows multiple containers to share the GPU's SMs concurrently. Required for SM% limits to be meaningful. Must run as a systemd service on the host.

sudo tee /etc/systemd/system/cuda-mps.service > /dev/null << 'EOF'

[Unit]

Description=CUDA MPS Control Daemon

After=nvidia-persistenced.service

[Service]

Type=forking

Environment=CUDA_VISIBLE_DEVICES=0

Environment=CUDA_MPS_PIPE_DIRECTORY=/tmp/nvidia-mps

Environment=CUDA_MPS_LOG_DIRECTORY=/tmp/nvidia-log

ExecStartPre=/usr/bin/nvidia-smi -pm 1

ExecStart=/usr/bin/nvidia-cuda-mps-control -d

ExecStop=/bin/bash -c "echo quit | /usr/bin/nvidia-cuda-mps-control"

Restart=on-failure

RestartSec=10

[Install]

WantedBy=multi-user.target

EOF

sudo systemctl daemon-reload

sudo systemctl enable --now cuda-mps

sudo systemctl status cuda-mps

# Expected: active (running)

ℹ INFO: echo get_server_list | nvidia-cuda-mps-control returning empty is NORMAL. MPS servers spawn on-demand when a CUDA process connects. Not a failure.

STEP 7    ZFS + NFS Persistent User Storage

Each user gets a dedicated ZFS dataset (15GB quota) as their persistent home directory, served via NFS and mounted into containers at /home/ubuntu.

### 7.1 — Install ZFS and NFS

sudo apt install -y zfsutils-linux nfs-kernel-server

### 7.2 — Production: ZFS Pool on Dedicated Partition

# On production nodes with dedicated storage partition:

# Replace /dev/nvme0n1p3 with your actual partition

sudo zpool create -f datapool /dev/nvme0n1p3

sudo zfs create datapool/users

# Verify pool

sudo zpool status

sudo zfs list

### 7.3 — POC Only: File-Backed Pool (space constrained)

# Only use if no dedicated partition available (POC workaround):

sudo mkdir -p /vg_containers

sudo truncate -s 20G /vg_containers/nas_pool.img

sudo zpool create -f datapool /vg_containers/nas_pool.img

sudo zfs create datapool/users

⚠ WARNING: File-backed ZFS pool is for POC only. Production must use a dedicated partition for correct performance and reliability.

### 7.4 — Create User Volumes

# Production: 15GB quota per user

# POC: 5GB quota (space constraint)

QUOTA=15G   # change to 5G for POC

# Create test users (replace with real user IDs in production)

for i in 1 2 3 4; do

sudo zfs create -o quota=$QUOTA datapool/users/testuser$i

done

# Verify quotas

sudo zfs get quota datapool/users/testuser1

### 7.5 — NFS Export and Mount

# Configure NFS export

# POC: loopback (127.0.0.1)

# Production: use NAS hostname, not IP

echo '/datapool/users 127.0.0.1(rw,sync,no_subtree_check,no_root_squash)' | \

sudo tee -a /etc/exports

sudo exportfs -ra

sudo systemctl restart nfs-kernel-server

# Mount NFS

sudo mkdir -p /mnt/nfs/users

sudo mount -t nfs4 127.0.0.1:/datapool/users /mnt/nfs/users

# Make mount persistent

echo '127.0.0.1:/datapool/users /mnt/nfs/users nfs4 defaults 0 0' | \

sudo tee -a /etc/fstab

# Set ownership (uid 1000 = ubuntu user inside container)

for i in 1 2 3 4; do

sudo chown -R 1000:1000 /mnt/nfs/users/testuser$i

done

# Verify

ls -la /mnt/nfs/users/

sudo zfs get quota datapool/users/testuser1

ℹ INFO: In production: replace 127.0.0.1 with nas.internal hostname. Use abstract user IDs as path components (e.g., /datapool/users/u_8f3a2c) rather than usernames to avoid leaking user identity in df output.

STEP 8    lxcfs (Resource Visibility Spoofing)

lxcfs is a FUSE filesystem that intercepts /proc reads and returns container-cgroup-aware values. Without it, containers see the host's full 64GB RAM and 32-core CPU in free -h, KDE System Settings, and all tools that read /proc.

sudo apt install -y lxcfs

sudo systemctl enable --now lxcfs

sudo systemctl status lxcfs

# Expected: active (running)

# Verify all required files are available

ls /var/lib/lxcfs/proc/

# Must see: cpuinfo meminfo stat uptime loadavg diskstats swaps

ls /var/lib/lxcfs/sys/devices/system/cpu/

# Must see: cpu0 cpu1 ... online possible present offline isolated

ℹ INFO: lxcfs must be running on the host before launching any containers. Add to all node startup checks. If lxcfs is not running, containers will show host specs to users.

| Resource | Without lxcfs | With lxcfs + correct flags | Method |
| --- | --- | --- | --- |
| RAM (free -h) | Shows host 62GB | Shows container limit (8GB) | lxcfs /proc/meminfo mount |
| RAM (KDE System Info) | Shows host 62GB | Shows container limit (8GB) | fake_sysconf.so interceptor (Step 10) |
| CPU count (nproc) | Shows host 32 | Shows container cores (4) | lxcfs /proc/cpuinfo + --cpuset-cpus |
| CPU (KDE System Info) | Shows host 32 cores | Hidden / correct count | /sys/devices/system/cpu static files (Step 10) |
| Load average | Shows host load | Shows container load | lxcfs /proc/loadavg mount |
| Uptime | Shows host uptime | Shows container uptime | lxcfs /proc/uptime mount |

STEP 9    Build fake_sysconf.so (RAM Fix for KDE)

KDE reads RAM via sysconf(_SC_PHYS_PAGES) syscall — not /proc/meminfo. lxcfs cannot intercept syscalls, only file reads. This tiny LD_PRELOAD library intercepts the sysconf call and returns the container-correct RAM value.

cat > /tmp/fake_sysconf.c << 'EOF'

#define _GNU_SOURCE

#include <unistd.h>

#include <dlfcn.h>

#include <stdlib.h>

#include <string.h>

// Reads CONTAINER_MEMORY_LIMIT_BYTES env var set at container launch

// Returns correct page count matching the container's RAM limit

long sysconf(int name) {

static long (*real_sysconf)(int) = NULL;

if (!real_sysconf)

real_sysconf = dlsym(RTLD_NEXT, "sysconf");

if (name == _SC_PHYS_PAGES || name == _SC_AVPHYS_PAGES) {

const char *limit = getenv("CONTAINER_MEMORY_LIMIT_BYTES");

if (limit) {

long page_size = real_sysconf(_SC_PAGESIZE);

long limit_bytes = atol(limit);

long pages = limit_bytes / page_size;

if (name == _SC_AVPHYS_PAGES)

return (long)(pages * 0.85);

return pages;

}

}

return real_sysconf(name);

}

EOF

# Compile

gcc -shared -fPIC -o /tmp/fake_sysconf.so /tmp/fake_sysconf.c -ldl

# Install

sudo cp /tmp/fake_sysconf.so /usr/lib/fake_sysconf.so

# Verify

ls -la /usr/lib/fake_sysconf.so

# Expected: ~16K

ℹ INFO: CONTAINER_MEMORY_LIMIT_BYTES is passed as an env var at container launch time (e.g., 8589934592 for 8GB). The library reads this at runtime so no recompilation is needed per container.

🔴 CRITICAL: Do NOT add fake_sysconf.so to selkies-gstreamer's LD_PRELOAD — it causes exit 139 (SIGSEGV). Only load it in the entrypoint (for KDE) and terminal sessions (via bash.bashrc).

STEP 10    Prepare All Host-Side Container Files

These files must exist on the host before launching any container. They are bind-mounted into each container at launch. Run this entire block once per node setup.

### 10.1 — Pull the Selkies Image

docker pull ghcr.io/selkies-project/nvidia-egl-desktop:latest

docker images | grep selkies

# Note: image is Ubuntu 24.04 internally (NOT 22.04)

# Default user inside: ubuntu (uid 1000)

### 10.2 — nvidia-smi Wrapper

cat > /tmp/nvidia-smi-wrapper << 'WRAPPER_EOF'

#!/bin/bash

# Reads enforced VRAM limit from CUDA_DEVICE_MEMORY_LIMIT_0 env var

# Replaces total VRAM in nvidia-smi output with container's actual limit

get_vram_limit_mib() {

if [ -n "$CUDA_DEVICE_MEMORY_LIMIT_0" ]; then

VAL="${CUDA_DEVICE_MEMORY_LIMIT_0,,}"

if [[ "$VAL" == *"m" ]]; then

echo "${VAL//m/}"

elif [[ "$VAL" == *"g" ]]; then

echo $(( ${VAL//g/} * 1024 ))

fi

return

fi

python3 -c "

import struct

with open('/tmp/cudevshr.cache','rb') as f:

d=f.read(4096)

print(struct.unpack_from('<Q',d,1600)[0]//(1024*1024))

" 2>/dev/null || echo "0"

}

# Pass through non-display commands unchanged

case "$*" in

*"-pm"*|*"--persistence-mode"*|*"-pl"*|*"--power-limit"*|\

*"--query-gpu"*|*"--format"*|*"-i "*|*"--id"*)

exec env -u LD_PRELOAD /usr/bin/nvidia-smi.real "$@"

;;

esac

LIMIT_MIB=$(get_vram_limit_mib)

REAL_OUT=$(env -u LD_PRELOAD /usr/bin/nvidia-smi.real "$@" 2>&1)

if [ -n "$LIMIT_MIB" ] && [ "$LIMIT_MIB" -gt "0" ] 2>/dev/null; then

echo "$REAL_OUT" | sed "s|/ [0-9]*MiB|/ ${LIMIT_MIB}MiB|g"

else

echo "$REAL_OUT"

fi

WRAPPER_EOF

chmod +x /tmp/nvidia-smi-wrapper

### 10.3 — passwd Wrapper

cat > /tmp/passwd-wrapper << 'EOF'

#!/bin/bash

exec env -u LD_PRELOAD /usr/bin/passwd.real "$@"

EOF

chmod +x /tmp/passwd-wrapper

### 10.4 — bash.bashrc with HAMi + sysconf Injection

# Get original bashrc from image

docker run --rm --entrypoint cat \

ghcr.io/selkies-project/nvidia-egl-desktop:latest \

/etc/bash.bashrc > /tmp/bash.bashrc.orig

# Append both interceptors

cat >> /tmp/bash.bashrc.orig << 'EOF'

# LaaS resource interceptors — applied to all bash terminal sessions

# HAMi: CUDA VRAM/SM limits  |  fake_sysconf: correct RAM in tools

if [ "${HAMI_INJECTED}" != "1" ]; then

export LD_PRELOAD="/usr/lib/libvgpu.so /usr/lib/fake_sysconf.so"

export HAMI_INJECTED=1

export SYSCONF_INJECTED=1

mkdir -p /tmp/vgpulock 2>/dev/null

fi

EOF

chmod 644 /tmp/bash.bashrc.orig

⚠ WARNING: profile.d scripts are NOT sourced in Konsole — it opens non-login interactive shells. /etc/bash.bashrc is the correct injection point. Do not use profile.d.

### 10.5 — supervisord Configuration

ℹ INFO: Critical rules: entrypoint gets fake_sysconf.so only (not libvgpu.so — VGL will crash). selkies-gstreamer gets libvgpu.so only (not fake_sysconf.so — segfault exit 139). User terminals get both via bash.bashrc.

cat > /tmp/supervisord-hami.conf << 'SUPERVISORD_EOF'

[unix_http_server]

file=/tmp/supervisor.sock

chmod=0700

[supervisord]

logfile=/tmp/supervisord.log

logfile_maxbytes=5MB

logfile_backups=0

loglevel=info

pidfile=/tmp/supervisord.pid

childlogdir=/tmp

nodaemon=true

[rpcinterface:supervisor]

supervisor.rpcinterface_factory = supervisor.rpcinterface:make_main_rpcinterface

[supervisorctl]

serverurl=unix:///tmp/supervisor.sock

[include]

files = /etc/supervisor/conf.d/*.conf

# fake_sysconf.so only — NO libvgpu.so (VGL crash)

[program:entrypoint]

command=bash -c "LD_PRELOAD=/usr/lib/fake_sysconf.so dbus-run-session -- /etc/entrypoint.sh"

stdout_logfile=/tmp/entrypoint.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=1

[program:dbus]

command=bash -c "mkdir -pm700 \"${XDG_RUNTIME_DIR}\"; chown -f \"$(id -nu):$(id -ng)\" \"${XDG_RUNTIME_DIR}\"; chmod -f 700 \"${XDG_RUNTIME_DIR}\"; dbus-daemon --system --nofork --nosyslog --nopidfile --address=\"${DBUS_SYSTEM_BUS_ADDRESS}\""

environment=DISPLAY="%(ENV_DISPLAY)s",XDG_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",DBUS_SYSTEM_BUS_ADDRESS="%(ENV_DBUS_SYSTEM_BUS_ADDRESS)s"

stdout_logfile=/tmp/dbus.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=1

# libvgpu.so only — NO fake_sysconf.so (exit 139 segfault)

[program:selkies-gstreamer]

command=bash -c "if [ \"$(echo ${KASMVNC_ENABLE} | tr '[:upper:]' '[:lower:]')\" != \"true\" ]; then LD_PRELOAD=/usr/lib/libvgpu.so /etc/selkies-gstreamer-entrypoint.sh; else sleep infinity; fi"

stdout_logfile=/tmp/selkies-gstreamer-entrypoint.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=20

[program:kasmvnc]

command=bash -c "if [ \"$(echo ${KASMVNC_ENABLE} | tr '[:upper:]' '[:lower:]')\" = \"true\" ]; then /etc/kasmvnc-entrypoint.sh; else sleep infinity; fi"

stdout_logfile=/tmp/kasmvnc-entrypoint.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=20

[program:nginx]

command=bash -c "until nc -z localhost ${SELKIES_PORT:-9081}; do sleep 0.5; done; /usr/sbin/nginx -g \"daemon off;\""

stdout_logfile=/tmp/nginx.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

priority=30

[group:pipewire-group]

program=pipewire,wireplumber,pipewire-pulse

priority=10

[program:pipewire]

command=bash -c "until [ -S \"/tmp/.X11-unix/X${DISPLAY#*:}\" ]; do sleep 0.5; done; dbus-run-session -- /usr/bin/pipewire"

environment=PIPEWIRE_LATENCY="128/48000",DISPLAY="%(ENV_DISPLAY)s",DISABLE_RTKIT="y",XDG_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",DBUS_SYSTEM_BUS_ADDRESS="%(ENV_DBUS_SYSTEM_BUS_ADDRESS)s",PIPEWIRE_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",PULSE_RUNTIME_PATH="%(ENV_XDG_RUNTIME_DIR)s/pulse"

stdout_logfile=/tmp/pipewire.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

[program:wireplumber]

command=bash -c "until [ \"$(echo ${XDG_RUNTIME_DIR}/pipewire-*.lock)\" != \"${XDG_RUNTIME_DIR}/pipewire-*.lock\" ]; do sleep 0.5; done; dbus-run-session -- /usr/bin/wireplumber"

environment=PIPEWIRE_LATENCY="128/48000",DISPLAY="%(ENV_DISPLAY)s",DISABLE_RTKIT="y",XDG_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",DBUS_SYSTEM_BUS_ADDRESS="%(ENV_DBUS_SYSTEM_BUS_ADDRESS)s",PIPEWIRE_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",PULSE_RUNTIME_PATH="%(ENV_XDG_RUNTIME_DIR)s/pulse"

stdout_logfile=/tmp/wireplumber.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

[program:pipewire-pulse]

command=bash -c "until [ \"$(echo ${XDG_RUNTIME_DIR}/pipewire-*.lock)\" != \"${XDG_RUNTIME_DIR}/pipewire-*.lock\" ]; do sleep 0.5; done; dbus-run-session -- /usr/bin/pipewire-pulse"

environment=PIPEWIRE_LATENCY="128/48000",DISPLAY="%(ENV_DISPLAY)s",DISABLE_RTKIT="y",XDG_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",DBUS_SYSTEM_BUS_ADDRESS="%(ENV_DBUS_SYSTEM_BUS_ADDRESS)s",PIPEWIRE_RUNTIME_DIR="%(ENV_XDG_RUNTIME_DIR)s",PULSE_RUNTIME_PATH="%(ENV_XDG_RUNTIME_DIR)s/pulse"

stdout_logfile=/tmp/pipewire-pulse.log

stdout_logfile_maxbytes=5MB

stdout_logfile_backups=0

redirect_stderr=true

stopasgroup=true

stopsignal=INT

autostart=true

autorestart=true

SUPERVISORD_EOF

### 10.6 — SM% Calculator Script

sudo tee /usr/local/bin/calc-sm-percent << 'EOF'

#!/bin/bash

# Usage: calc-sm-percent <container_vram_gb> <total_gpu_vram_gb>

# SM% = VRAM_allocated / total_VRAM * 100 (proportional allocation)

# RTX 4090 POC:  total=24   RTX 5090 Production: total=32

CONTAINER_VRAM=${1:?Usage: calc-sm-percent <container_vram_gb> <total_gpu_vram_gb>}

TOTAL_VRAM=${2:?Usage: calc-sm-percent <container_vram_gb> <total_gpu_vram_gb>}

PCT=$(( (CONTAINER_VRAM * 100) / TOTAL_VRAM ))

[ $PCT -lt 5 ] && PCT=5

echo $PCT

EOF

sudo chmod +x /usr/local/bin/calc-sm-percent

# Verify

echo 'RTX 4090 (24GB pool):'

for v in 4 8 12 16 24; do

echo "  ${v}GB → $(calc-sm-percent $v 24)% SMs"

done

echo 'RTX 5090 (32GB pool):'

for v in 4 8 16 32; do

echo "  ${v}GB → $(calc-sm-percent $v 32)% SMs"

done

### 10.7 — Per-Container vgpulock Directories

for i in 1 2 3 4; do

sudo mkdir -p /tmp/vgpulock-$i

sudo chmod 777 /tmp/vgpulock-$i

done

ls -la /tmp/ | grep vgpulock

### 10.8 — Static CPU Sys Files (per container)

lxcfs does not filter /sys/devices/system/cpu by cpuset. Static files with exact correct cpuset ranges are created and mounted per container. KDE reads these to report CPU count.

for i in 1 2 3 4; do

CPUSET_START=$(( ($i-1)*4 ))

CPUSET_END=$(( ($i-1)*4+3 ))

CPUSET="${CPUSET_START}-${CPUSET_END}"

mkdir -p /tmp/container-$i-cpu

echo "$CPUSET" > /tmp/container-$i-cpu/online

echo "$CPUSET" > /tmp/container-$i-cpu/present

echo "$CPUSET" > /tmp/container-$i-cpu/possible

echo ""         > /tmp/container-$i-cpu/offline

echo ""         > /tmp/container-$i-cpu/isolated

chmod 444 /tmp/container-$i-cpu/*

echo "Container $i → cpuset: $CPUSET → $(cat /tmp/container-$i-cpu/online)"

done

### 10.9 — Final File Check

echo '=== Required host-side files ==='

ls -la /usr/lib/libvgpu.so

ls -la /usr/lib/fake_sysconf.so

ls -la /tmp/nvidia-smi-wrapper

ls -la /tmp/passwd-wrapper

ls -la /tmp/bash.bashrc.orig

ls -la /tmp/supervisord-hami.conf

ls -la /var/lib/lxcfs/proc/meminfo

ls -la /tmp/vgpulock-1 /tmp/vgpulock-2 /tmp/vgpulock-3 /tmp/vgpulock-4

ls -la /tmp/container-1-cpu/online

ls -la /mnt/nfs/users/

echo '=== All good if no errors ==='

STEP 11    Launch Containers

### 11.1 — Port Assignment Scheme

| Slot | NGINX_PORT (Web UI) | SELKIES_PORT | METRICS_PORT | DISPLAY | cpuset |
| --- | --- | --- | --- | --- | --- |
| 1 | 8081 | 9081 | 19081 | :20 | 0-3 |
| 2 | 8082 | 9082 | 19082 | :21 | 4-7 |
| 3 | 8083 | 9083 | 19083 | :22 | 8-11 |
| 4 | 8084 | 9084 | 19084 | :23 | 12-15 |
| N | 8080+N | 9080+N | 19080+N | :19+N | (N-1)*4 to (N-1)*4+3 |

### 11.2 — Single Container Launch (Template)

# Set variables

i=1

NGINX_P=8081

SELKIES_P=9081

METRICS_P=19081

DISPLAY_N=20

CPUSET=0-3

VRAM_GB=4                              # Adjust per tier

TOTAL_VRAM_GB=24                       # 24 for RTX 4090, 32 for RTX 5090

SM_PCT=$(calc-sm-percent $VRAM_GB $TOTAL_VRAM_GB)

MEM_BYTES=$((8*1024*1024*1024))        # 8GB RAM in bytes

SESSION_HOST="ws-$(openssl rand -hex 4)"

TOKEN=$(openssl rand -hex 16)

echo "URL: http://100.100.66.101:$NGINX_P | token: $TOKEN | host: $SESSION_HOST"

docker run -d \

--name selkies-$i \

--hostname $SESSION_HOST \

--gpus all \

--cpus=4 \

--cpuset-cpus=$CPUSET \

--memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC \

-e DISPLAY=:$DISPLAY_N \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=$TOKEN \

-e NGINX_PORT=$NGINX_P \

-e SELKIES_PORT=$SELKIES_P \

-e SELKIES_METRICS_HTTP_PORT=$METRICS_P \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 \

-e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 \

-e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0=${VRAM_GB}192m \

-e CUDA_DEVICE_SM_LIMIT=$SM_PCT \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="0=${VRAM_GB}G" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=$SM_PCT \

-e CUDA_MPS_ENABLE_PER_CTX_DEVICE_MULTIPROCESSOR_PARTITIONING=1 \

-e CONTAINER_MEMORY_LIMIT_BYTES=$MEM_BYTES \

-e SELKIES_TURN_HOST=<YOUR_TURN_SERVER_IP> \

-e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=<TURN_USER> \

-e SELKIES_TURN_PASSWORD=<TURN_PASS> \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /mnt/nfs/users/testuser$i:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /usr/lib/fake_sysconf.so:/usr/lib/fake_sysconf.so \

-v /tmp/vgpulock-$i:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

-v /tmp/container-$i-cpu:/sys/devices/system/cpu:ro \

ghcr.io/selkies-project/nvidia-egl-desktop:latest

⚠ WARNING: --network=host is POC only. In production with Cloudflare Tunnel, use --network=bridge and -p $NGINX_P:8080. Change CUDA_NVRTC_ARCH=89 to 100 on RTX 5090 production nodes.

### 11.3 — All 4 Containers (Loop)

TOTAL_VRAM_GB=24   # Change to 32 on RTX 5090

for i in 1 2 3 4; do

NGINX_P=$((8080 + i))

SELKIES_P=$((9080 + i))

METRICS_P=$((19080 + i))

DISPLAY_N=$((19 + i))

CPUSET="$(( ($i-1)*4 ))-$(( ($i-1)*4+3 ))"

VRAM_GB=$([ $i -le 2 ] && echo '4' || echo '8')

SM_PCT=$(calc-sm-percent $VRAM_GB $TOTAL_VRAM_GB)

MEM_BYTES=$((8*1024*1024*1024))

SESSION_HOST="ws-$(openssl rand -hex 4)"

TOKEN=$(openssl rand -hex 16)

echo "Container $i → VRAM: ${VRAM_GB}GB | SM: ${SM_PCT}% | cpuset: $CPUSET | port: $NGINX_P | token: $TOKEN"

docker run -d \

--name selkies-test-$i \

--hostname $SESSION_HOST \

--gpus all --cpus=4 --cpuset-cpus=$CPUSET --memory=8g \

--ipc=host --network=host --tmpfs /dev/shm:rw \

-e TZ=UTC -e DISPLAY=:$DISPLAY_N \

-e DISPLAY_SIZEW=1920 -e DISPLAY_SIZEH=1080 -e DISPLAY_REFRESH=60 \

-e SELKIES_ENCODER=nvh264enc \

-e SELKIES_ENABLE_BASIC_AUTH=true \

-e SELKIES_BASIC_AUTH_PASSWORD=$TOKEN \

-e NGINX_PORT=$NGINX_P -e SELKIES_PORT=$SELKIES_P \

-e SELKIES_METRICS_HTTP_PORT=$METRICS_P \

-e PASSWD=mypasswd \

-e CUDA_VISIBLE_DEVICES=0 -e CUDA_NVRTC_ARCH=89 \

-e __NV_PRIME_RENDER_OFFLOAD=1 -e __GLX_VENDOR_LIBRARY_NAME=nvidia \

-e CUDA_DEVICE_MEMORY_LIMIT_0=${VRAM_GB}192m \

-e CUDA_DEVICE_SM_LIMIT=$SM_PCT \

-e CUDA_MPS_PINNED_DEVICE_MEM_LIMIT="0=${VRAM_GB}G" \

-e CUDA_MPS_ACTIVE_THREAD_PERCENTAGE=$SM_PCT \

-e CUDA_MPS_ENABLE_PER_CTX_DEVICE_MULTIPROCESSOR_PARTITIONING=1 \

-e CONTAINER_MEMORY_LIMIT_BYTES=$MEM_BYTES \

-e SELKIES_TURN_HOST=<TURN_IP> -e SELKIES_TURN_PORT=3478 \

-e SELKIES_TURN_USERNAME=<TURN_USER> -e SELKIES_TURN_PASSWORD=<TURN_PASS> \

-e SELKIES_TURN_PROTOCOL=tcp \

-v /mnt/nfs/users/testuser$i:/home/ubuntu \

-v /usr/lib/libvgpu.so:/usr/lib/libvgpu.so \

-v /usr/lib/fake_sysconf.so:/usr/lib/fake_sysconf.so \

-v /tmp/vgpulock-$i:/tmp/vgpulock \

-v /usr/bin/nvidia-smi:/usr/bin/nvidia-smi.real \

-v /tmp/nvidia-smi-wrapper:/usr/bin/nvidia-smi \

-v /usr/bin/passwd:/usr/bin/passwd.real \

-v /tmp/passwd-wrapper:/usr/bin/passwd \

-v /tmp/supervisord-hami.conf:/etc/supervisord.conf \

-v /tmp/bash.bashrc.orig:/etc/bash.bashrc \

-v /var/lib/lxcfs/proc/cpuinfo:/proc/cpuinfo:ro \

-v /var/lib/lxcfs/proc/meminfo:/proc/meminfo:ro \

-v /var/lib/lxcfs/proc/stat:/proc/stat:ro \

-v /var/lib/lxcfs/proc/uptime:/proc/uptime:ro \

-v /var/lib/lxcfs/proc/loadavg:/proc/loadavg:ro \

-v /var/lib/lxcfs/proc/diskstats:/proc/diskstats:ro \

-v /var/lib/lxcfs/proc/swaps:/proc/swaps:ro \

-v /tmp/container-$i-cpu:/sys/devices/system/cpu:ro \

ghcr.io/selkies-project/nvidia-egl-desktop:latest

done

STEP 12    Validation Checklist

Run after every fresh setup. All items must pass before accepting user traffic.

### 12.1 — Host-Level Checks

# All containers running

docker ps --format 'table {{.Names}}\t{{.Status}}'

# Ports listening

ss -tlnp | grep nginx

# Expected: 0.0.0.0:8081, 8082, 8083, 8084

# MPS running

sudo systemctl status cuda-mps | grep Active

# lxcfs running

sudo systemctl status lxcfs | grep Active

# NFS mounted

mount | grep nfs

### 12.2 — Per-Container Automated Check

for i in 1 2 3 4; do

echo "--- Container $i ---"

docker exec selkies-test-$i bash -c '

echo "hostname:  $(hostname)"

echo "nproc:     $(nproc)"

echo "ram:       $(free -h | awk "/^Mem/ {print \$2}")"

echo "cpu_online: $(cat /sys/devices/system/cpu/online)"

echo "ld_preload: $LD_PRELOAD"

'

VRAM_GB=$([ $i -le 2 ] && echo '4' || echo '8')

docker exec selkies-test-$i python3 -c "

import struct

with open('/tmp/cudevshr.cache','rb') as f: d=f.read(4096)

limit=struct.unpack_from('<Q',d,1600)[0]

sm=struct.unpack_from('<Q',d,1728)[0]

print(f'  vram_limit: {limit/1024**3:.1f}GB  sm_limit: {sm}%')" 2>/dev/null || echo '  cache: not yet created (normal)'

done

### 12.3 — In-Container Validation (run in browser Konsole)

| Command | Expected Result | What it validates |
| --- | --- | --- |
| hostname | ubuntu@ws-xxxxxxxx | Random hostname, no host leakage |
| nproc | 4 | CPU cpuset enforced and visible |
| free -h | Mem: 8.0Gi | RAM limit enforced and visible |
| cat /sys/devices/system/cpu/online | 0-3 (or 4-7, 8-11, 12-15) | CPU sys files correct |
| echo $LD_PRELOAD | /usr/lib/libvgpu.so /usr/lib/fake_sysconf.so | Both interceptors loaded |
| nvidia-smi (Memory-Usage column) | xxMiB / 4096MiB | VRAM limit displayed correctly |
| KDE → About this System → Memory | 8.0 GiB | fake_sysconf.so working in KDE |
| KDE → About this System → Processor | Not shown / 4 cores | CPU sys files working in KDE |
| df -h /home/ubuntu | ~15G (or 5G POC) | NFS home mounted correctly |

### 12.4 — HAMi VRAM Enforcement Test (once torch is installed)

# Install torch (first time only — persists in NFS home)

pip install torch --index-url https://download.pytorch.org/whl/cu128 -q --break-system-packages

python3 -c "

import torch

total = torch.cuda.get_device_properties(0).total_memory

print(f'Reported VRAM: {total/1024**3:.1f}GB')   # Must match tier limit

try:

t = torch.zeros(int(total*1.5/4), dtype=torch.float32, device='cuda')

print('FAIL: over-limit allocation succeeded')

except RuntimeError:

print('PASS: over-limit allocation blocked')

try:

t = torch.zeros(int(total*0.7/4), dtype=torch.float32, device='cuda')

print('PASS: normal allocation succeeded')

except RuntimeError:

print('FAIL: normal allocation blocked')

"

# Appendix A — Tier Resource Configurations

The orchestrator maps subscription tier to resource parameters at container launch time.

| Tier | CPUs | RAM | VRAM | SM% (RTX 4090) | SM% (RTX 5090) | Session Type |
| --- | --- | --- | --- | --- | --- | --- |
| Starter | 2 | 4GB | None | — | — | Stateful GUI |
| Standard | 4 | 8GB | None | — | — | Stateful GUI |
| Pro | 4 | 8GB | 4GB | 16% | 12% | Stateful GUI |
| Power | 6 | 16GB | 8GB | 33% | 25% | Stateful GUI |
| Max | 8 | 24GB | 16GB | 66% | 50% | Stateful GUI |
| Full Node | 16 | 64GB | 24/32GB | 100% | 100% | Stateful GUI |
| Ephemeral CPU | 2-4 | 4-8GB | None | — | — | CLI/Jupyter |
| Ephemeral GPU | 4 | 8GB | 4-8GB | 16-33% | 12-25% | CLI/Jupyter |

# Appendix B — Known Limitations

| Limitation | Cause | Mitigation |
| --- | --- | --- |
| nvidia-smi shows full GPU VRAM | NVML queries kernel driver — bypasses all user-space interception | nvidia-smi wrapper reads CUDA_DEVICE_MEMORY_LIMIT_0 env var and replaces total VRAM in output |
| Memory bandwidth shared | Consumer GPU (no MIG support on RTX) | SM proportional allocation minimises contention. Disclosed in ToS as shared-GPU limitation |
| --network=host (POC) | WebRTC requires direct port access over Tailscale | Production: --network=bridge + Cloudflare Tunnel |
| df shows overlay + NFS paths | Kernel-level mounts always visible | Use abstract NFS paths with no user-identifying info |
| lxcfs /sys filtering | lxcfs doesn't filter /sys by cpuset | Static per-container CPU files mounted at /sys/devices/system/cpu |
| Container startup ~15-25s | KDE Plasma initialisation time | Pre-warm pool for production UX |

# Appendix C — Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| selkies-gstreamer exit 139 in logs | Both libvgpu.so AND fake_sysconf.so in selkies-gstreamer LD_PRELOAD | Remove fake_sysconf.so from selkies-gstreamer — libvgpu.so only |
| LD_PRELOAD not set in Konsole | Injected in profile.d instead of bash.bashrc | Ensure /etc/bash.bashrc has the injection block |
| VRAM shows 23.5GB in ctypes test | ctypes.CDLL uses RTLD_LOCAL — bypasses LD_PRELOAD | Use torch for VRAM tests. ctypes always shows physical GPU. |
| nvidia-smi segfault | LD_PRELOAD + NVML conflict | Ensure nvidia-smi wrapper strips LD_PRELOAD before calling real binary |
| KDE RAM still shows host value | CONTAINER_MEMORY_LIMIT_BYTES env var missing | Verify -e CONTAINER_MEMORY_LIMIT_BYTES=$MEM_BYTES in docker run |
| nproc shows 32 | --cpuset-cpus not in docker run | Add --cpuset-cpus=0-3 (or appropriate range) to docker run |
| KDE CPU shows host cores | /tmp/container-$i-cpu not mounted | Verify -v /tmp/container-$i-cpu:/sys/devices/system/cpu:ro in docker run |
| Container not accessible in browser | selkies-gstreamer in FATAL state | Check docker logs <name> --tail 30 — look for crash reason |
| HAMi not enforcing limits | libvgpu.so at wrong path | Must be /usr/lib/libvgpu.so — not /usr/local/lib/ |

LaaS Node Setup Guide v1.0 — End of Document